SCG 007C

Operating Limits

V05

| ## Version | ## Initials | ## Update | ## Date |
| --- | --- | --- | --- |
| V00 | MT | Initial draft | 10.12.20 |
| V01 | FV | Main engine and gearbox parameters added | 11.02.21 |
| V02 | LB | Main suspension, brakes and steering parameters added | 23.02.21 |
| V03 | LB | FTE and RTE travel limits and damper temperatures added | 24.02.21 |
| V04 | LB | Wheel bearing grease temperature limit added | 01.03.21 |
| V05 | LB | Rockers and driveshaft torque sensor max temp added | 11.03.21 |

General Information

Updates for each version are marked in magenta.

Definitions:

- The normal operating limits are acceptable for continuous operation.

- The warning limit is the threshold at which investigation is required and, if applicable, a warning is triggered in the data.

- The absolute limit is the threshold at which the car operation must be modified or a component or system must be inspected following the occurrence.

| Measure; Units; Normal limit; Warning limit; Maximum limit; Notes. |
| --- |
| FRONT SUSPENSION |  |  |  |  |  |
| Pushrod load | kN | 30 | 43 | 60 | Inspect welding if warning limit reached. Inspect entire suspension if limit reached. |
| Lateral damper bump travel (from full extended position) | mm |  |  | 65.2 |  |
| Front third element travel (from full extended position) | mm |  |  | 86.97 |  |
| Lateral damper temperature | 0C |  | 120 |  |  |
| Third element temperature | 0C |  | 120 |  |  |
| Wheel bearing grease temperature | 0C |  |  | 150 | Inspect and send it to Pankl for rebuild |
| Rocker bearing temperature | 0C |  |  | 120 | Replace if max limit is reached |
| Front rim temperature | 0C |  | 150 |  | Inspect rim if max limit reached |
| STEERING |  |  |  |  |  |
| Column torque | Nm | 50 | 50 | TBA | Inspect column if max limit reached |
| Rack travel | mm |  |  | 44 |  |
| REAR SUSPENSION |  |  |  |  |  |
| Pushrod load | kN | 53 | 60 | 90 | Inspect welding if warning limit reached. Inspect entire suspension if limit reached. |
| Lateral damper bump travel (from full extended position) | mm |  |  | 80.2 |  |
| Front third element travel (from full extended position) | mm |  |  | 86.97 |  |
| Lateral damper temperature | 0C |  | 120 |  |  |
| Third element temperature | 0C |  | 120 |  |  |
| Wheel bearing grease temperature | 0C |  |  | 150 | Inspect and send it to Pankl for rebuild |
| Rear rim temperature |  |  | 150 |  |  |
| Driveshaft torque | Nm |  |  | 5000 | Measure twist and check sensor calibration if max limit reached |
| Driveshaft torque sensor temperature | 0C |  |  | 110 | Replace if max limit is reached |
| Rocker bearing temperature |  |  |  | 177 | Replace if max limit is reached |
| BRAKES |  |  |  |  |  |
| Front brake pressure | barA | 100 | 120 | TBA |  |
| Rear brake pressure | barA | 100 | 120 | TBA |  |
| Front Master cylinder travel | mm | 16.7 |  | 30 | Check brake system if warning limit reached |
| Rear Master cylinder travel | mm | 13.7 |  | 30 | Check brake system if warning limit reached |
| Front disc temperature | 0C | 650 | 650 |  |  |
| Rear disc temperature | 0C | 650 | 650 |  |  |
| Front caliper temperature | 0C | 150 | 150 | 180 | Inspect caliper if max limit reached |
| Rear caliper temperature | 0C | 150 | 150 | 180 | Inspect caliper if max limit reached |
| Front brake wear (minimum pad and disc stack) | mm |  |  | 49 | Change brake material if max limit reached |
| Rear brake wear (minimum pad and disc stack) | mm |  |  | 49 | Change brakes material max limit reached |
| ENGINE |  |  |  |  |  |
| Max Engine speed | rpm | 7500 |  | >8000 |  |
| Max water temp out of engine (Tmot1-Tmot2) | 0C | 110 |  | >115 |  |
| Min water temp out of engine (Tmot1-Tmot2) | 0C | 60 | 20 | <10 |  |
| Max water pressure (Pmot-in) | barA | 4.0 |  | 4.5 |  |
| Min water pressure (Pmot-in) | barA | 0.7 |  | <0.50 |  |
| Max Header tank pressure (Pwat) | barA | 3 |  | 3.5 |  |
| Min Header tank pressure (Pwat) | barA | 0.5 |  | <0.2 |  |
| Max oil temperature (Toil) | 0C | 125 |  | 135 |  |
| Min oil temperature (Toil) | 0C | 60 | 20 | <10 |  |
| Oil pressure @1500 rpm (Poil) | barA | 1.5 |  | <0.5 @2000 rpm |  |
| Oil pressure @5000 rpm (Poil) | barA | 6.5 |  | <3.5 |  |
| Min Oil tank level (Oillvl) | ltrs | 2.0 |  | <1.5 |  |
| Max Oil tank level (Oillvl) | ltrs | 6.5 |  | >7.0 |  |
| Min Crank case pressure (Pcrank) | barA | 0.80 |  | <0.75 |  |
| Max Crank case pressure (Pcrank) | barA | 0.95 |  | >1.0 |  |
| Charge air temperature after CAC (T21_1-T21_2) | 0C | 45 |  | 50 |  |
| Charge air pressure after CAC (P21_1-P21_2) | barA | 2.5 |  | >3.0 |  |
| Plenum inlet temperature (Tinlet) | 0C | 45 |  | 50 |  |
| Min Plenum inlet pressure (P22_1-P22_2) | barA | 0.3 |  | <0.25 |  |
| Max Plenum inlet pressure (P22_1-P22_2) | barA | 2.3 |  | >2.5 |  |
| Min Barometric pressure (Pbaro) | barA | 0.85 |  | <0.75 |  |
| Max Turbo speed (nturbo) | rpm | 185000 |  | >185000 |  |
| Min fuel rail pressure (Prail_1-Prail_2) | barG | 60 |  | <60 |  |
| Max fuel rail pressure (Prail_1-Prail_2) | barG | 150 |  | >250 |  |
| Max fuel rail temperature (Trail1-Trail2) | 0C | 70 |  | >85 |  |
| Min PFI fuel rail pressure relative (Pfuel_1-Pfuel_2) | barG | 6.0 |  | <5.5 |  |
| Max fuel rail pressure (Pfuel_1-Pfuel_2) | barG | 8.0 |  | >8.5 |  |
| Max PFI fuel rail temperature (Tfuel1-Tfuel2) | 0C | 60 |  | >80 |  |
| Exhaust temperature (Texh1-Texh2) | 0C | 850 |  | >900 |  |
| Min lambda | - | 0.8 |  | <0.8 |  |
| Max lambda | - | 1.05 |  | >1.05 |  |
| Max Clutch wear travel | mm | 6 |  | >6 | Clutch to be replaced |
| Max clutch release travel | mm | 4.5 |  | >5 | Value to be evaluated from current locking position (variable with wear), check clutch in case of occurence |
| Oil catch tank level |  |  |  |  |  |
| GEARBOX |  |  |  |  |  |
| Max gearbox oil temperature | 0C | 100 | 110 |  |  |
| Min gearbox oil temperature | 0C | 90 |  | <65 | Full input torque can not be applied until the critical temperature is reached. Warming up with run up wheels permitted. |
| Max gearbox outlet pressure | barG | 2.0 |  |  | Due to scavenging nature of the system cornering and banking can surge the system, pressure drops can be tolerated by the system. The important thing is that the pressure picks up quickly afterwards and there is no increase in oil temperature as a result |
| Min gearbox outlet pressure | barG | 0.5 |  |  | Due to scavenging nature of the system cornering and banking can surge the system, pressure drops can be tolerated by the system. The important thing is that the pressure picks up quickly afterwards and there is no increase in oil temperature as a result |
| FUEL SYSTEM |  |  |  |  |  |
| Fuel cell pressure | barA | 1.2 |  |  |  |
| ELECTRICAL SYSTEM |  |  |  |  |  |
| Engine alternator temperature | 0C |  |  |  |  |
| Gearbox alternator temperature | 0C |  |  |  |  |
| Starter temperature | 0C |  |  |  |  |
| Gearbox actuator temperature | 0C |  |  |  |  |
| Powerbox temperature | 0C |  |  |  |  |
| ECU temperature | 0C |  |  |  |  |
| Battery temperature | 0C |  |  |  |  |
| COCKPIT |  |  |  |  |  |
| Cockpit temperature at ambient <25 0C | 0C | 30 | 30 | 32 | Max limit to be respected within 8 minutes after a pitstop |
| Cockpit temperature at ambient >25 0C | 0C | Ambient +5 | Ambient +5 | Ambient +7 | Max limit to be respected within 8 minutes after a pitstop |